ESP32智能设备开发：

- 基于ESP32的FreeRTOS原生能力

- 基于小智AI框架

- 基于乐鑫提供的新版人机交互框架( 类似小智AI)

- 前提准备：

- 小智AI源码: V2.2.4

- ESP-IDF: 	  V5.5.3

- 芯片:       ESP32-S3

- Flash:       16M

- PSRAM:      8M

- 整体策略:

- 以小智AI V2.2.4框架为主体

- 新增自定义板型

- 将I2C驱动移植( 修改为ESP-IDF V5 推荐新版I2C)

- 适配ES7310 +ES8311(录音+播放)

- 验证在线语音

- 接入LCD

- 接入摄像头

- 接入触摸屏

- MCP控制

- {

- “cmd”: “打开LED灯”

- “id”:  1

- “function”: led_on

- “arg”:[ bool]

- }

- 一、准备工程

- 下载连接:下载仓库 · ghosthack/xiaozhi-esp32 - Gitee.com

- 解压工程

- 导入VSCode

- 修改ESP-IDF版本、芯片型号、 串口

- 构建工程( 确保源码没有问题)

- 创建自定义板型

- main/boards/gec-esp32s3/

- |- config.h		//平台对应的IO引脚、句柄

- |- config.json

- |- gec_esp32s3_boards.cc

- |- gec_esp32s3_boards.h

- |- gec_audio_codec.cc

- |- readme.md

- 编写Json文件

{

"type": "gec_esp32s3",	//配置参数生效的板型

"target": "esp32s3",		//配置参数生效的芯片型号

"build": [

{

"name":"gec_esp32s3",

"sdkconfig_append": [

"CONFIG_ESPTOOLPY_FLASHSIZE_16MB=y"   //设置芯片flash大小:16MB

]

}

]

}

- 注册Kconfig板型

- main/kconfig.projbuild

- 2.1在文件中搜索choice BOARD_TYPE

- 2.2 添加以下代码

config BOARD_TYPE_GEC_ESP32S3

bool "GEC ESP32-S3 Custom Board"			//SDK编辑器中提示的文字

depends on IDF_TARGET_ESP32S3			//依赖芯片型号， 必须设置为ESP32S3型号，并重新构建工程后，才会在SDK编辑器中显示对应的板型

- 注册CMake板型目录

- main/CMakeLists.txt

  - 在现有的板型选择语句中添加一下代码

elseif(CONFIG_BOARD_TYPE_GEC_ESP32S3)

set(BOARD_TYPE "gec_esp32s3")	    #板型代码的文件夹

set(BUILTIN_TEXT_FONT font_puhui_basic_30_4)	//lvgl显示字体大小

set(BUILTIN_ICON_FONT font_awesome_30_4)		//lvgl显示字体大小

set(DEFAULT_EMOJI_COLLECTION twemoji_64)

- 验证板型菜单

  - 设置芯片信号 esp32s3

  - 编译代码

  - 打开SDK编辑器

  - 进入 Xiaozhi Assistant

- |- Board Type

- |- GEC ESP32-S3 Custom Board

- 编写板级硬件配置

- main/boards/gec_esp32s3/config.h

#pragma once

#include <driver/gpio.h>

#include <driver/i2c_types.h>

#include <driver/spi_master.h>

/*================ 公共I2C总线 ================*/

#define BOARD_I2C_PORT                  I2C_NUM_0

#define BOARD_I2C_SDA_GPIO              GPIO_NUM_1

#define BOARD_I2C_SCL_GPIO              GPIO_NUM_2

#define BOARD_I2C_FREQ_HZ               100000

/*================ I2C设备地址 ================*/

#define BOARD_PCA9557_ADDR              0x19

#define AUDIO_CODEC_ES8311_ADDR         (0x18<<1)

#define AUDIO_CODEC_ES7210_ADDR         (0x41<<1)

/*================ PCA9557引脚 ================*/

#define BOARD_PCA9557_LCD_CS_BIT        0

#define BOARD_PCA9557_PA_EN_BIT         1

#define BOARD_PCA9557_CAMERA_PWDN_BIT   2

/*================ 音频 ================*/

#define AUDIO_INPUT_SAMPLE_RATE         16000

#define AUDIO_OUTPUT_SAMPLE_RATE        16000

#define AUDIO_I2S_GPIO_MCLK             GPIO_NUM_38

#define AUDIO_I2S_GPIO_BCLK             GPIO_NUM_14

#define AUDIO_I2S_GPIO_WS               GPIO_NUM_13

#define AUDIO_I2S_GPIO_DIN              GPIO_NUM_12

#define AUDIO_I2S_GPIO_DOUT             GPIO_NUM_45

/* PA由PCA9557 IO1控制，不是ESP32 GPIO */

#define AUDIO_CODEC_PA_PIN              GPIO_NUM_NC

/*

* false：暂不提供扬声器回采参考通道。

* 基础对话正常后再调试AEC。

*/

#define AUDIO_INPUT_REFERENCE           false

/*================ LCD ================*/

#define DISPLAY_SPI_HOST                SPI2_HOST

#define DISPLAY_SPI_SCK_PIN             GPIO_NUM_41

#define DISPLAY_SPI_MOSI_PIN            GPIO_NUM_40

/* CS由PCA9557 IO0控制 */

#define DISPLAY_SPI_CS_PIN              GPIO_NUM_NC

#define DISPLAY_DC_PIN                  GPIO_NUM_39

#define DISPLAY_RESET_PIN               GPIO_NUM_NC

#define DISPLAY_BACKLIGHT_PIN           GPIO_NUM_42

#define DISPLAY_WIDTH                   480

#define DISPLAY_HEIGHT                  320

#define DISPLAY_OFFSET_X                0

#define DISPLAY_OFFSET_Y                0

#define DISPLAY_SWAP_XY                 true

#define DISPLAY_MIRROR_X                true

#define DISPLAY_MIRROR_Y                true

#define DISPLAY_INVERT_COLOR            true

#define DISPLAY_BACKLIGHT_OUTPUT_INVERT false

/*================ 按键 ================*/

/*

* 没有确定物理按键时暂时禁用。

* 后续确认按键GPIO后替换。

*/

#define BOOT_BUTTON_GPIO                GPIO_NUM_NC

- 编写完整的板机代码(C++编写)

- main/boards/gec_esp32s3/gec_esp32s3_board.cc

- 实现基本功能

  - 公共I2C接口

  - PCA9557

  - ES8311

  - ES7210

  - I2S

  - ST7789显示屏接口

  - LCD背光控制

  - 小智WIFI功能

#include "wifi_board.h"

#include "codecs/box_audio_codec.h"

#include "display/lcd_display.h"

#include "config.h"

#include "i2c_device.h"

#include <esp_check.h>

#include <esp_log.h>

#include <esp_lcd_panel_vendor.h>

#include <driver/i2c_master.h>

#include <driver/spi_common.h>

#define TAG "GecEsp32S3Board"

/*

* PCA9557寄存器：

* 0x00 Input Port

* 0x01 Output Port

* 0x02 Polarity Inversion

* 0x03 Configuration

*/

class GecPca9557 : public I2cDevice {

public:

GecPca9557(i2c_master_bus_handle_t bus, uint8_t address): I2cDevice(bus, address) {

/*

* IO7~IO3：输入

* IO2~IO0：输出

*/

WriteReg(0x03, 0xF8);

/*

* IO0 LCD_CS = 1，暂不选中LCD

* IO1 PA_EN = 0，关闭功放

* IO2 CAMERA_PWDN = 1，摄像头休眠

*/

output_state_ = 0x05;

WriteReg(0x01, output_state_);

}

void SetOutputState(uint8_t bit, bool level) {

if (bit > 7) {

ESP_LOGE(TAG, "Invalid PCA9557 bit: %u", bit);

return;

}

if (level) {

output_state_ |= static_cast<uint8_t>(1U << bit);

} else {

output_state_ &= static_cast<uint8_t>(~(1U << bit));

}

WriteReg(0x01, output_state_);

}

private:

uint8_t output_state_ = 0xFF;

};

/*

* 小智自带BoxAudioCodec：

* 输出使用ES8311

* 输入使用ES7210

*/

class GecAudioCodec : public BoxAudioCodec {

public:

//BoxAudioCodec 继承: 提供音频如何上传服务器   初始化I2S的设备

GecAudioCodec(i2c_master_bus_handle_t i2c_bus,GecPca9557* pca9557): BoxAudioCodec(

i2c_bus,

AUDIO_INPUT_SAMPLE_RATE,

AUDIO_OUTPUT_SAMPLE_RATE,

AUDIO_I2S_GPIO_MCLK,

AUDIO_I2S_GPIO_BCLK,

AUDIO_I2S_GPIO_WS,

AUDIO_I2S_GPIO_DOUT,

AUDIO_I2S_GPIO_DIN,

GPIO_NUM_NC,

AUDIO_CODEC_ES8311_ADDR,

AUDIO_CODEC_ES7210_ADDR,

AUDIO_INPUT_REFERENCE),

pca9557_(pca9557) {

}

void EnableOutput(bool enable) override {

/*

* 先操作Codec，再控制外部功放。

* 关闭时先关闭Codec输出，再拉低PA_EN。

*/

BoxAudioCodec::EnableOutput(enable);

pca9557_->SetOutputState( BOARD_PCA9557_PA_EN_BIT, enable);

}

private:

GecPca9557* pca9557_;

};

//板子的类 GecEsp32S3Board

class GecEsp32S3Board : public WifiBoard {

public:

//初始化函数: 对象创建时自动调用  GecEsp32S3Board gec_esp32s3;

GecEsp32S3Board() {

InitializeI2c();

InitializeSpi();

InitializeDisplay();

ESP_LOGI(TAG, "GEC ESP32-S3 board initialized");

}

//获取音频编码器的句柄(扬声器)

//返回AudioCodec对象指针

//由小智AI框架调用， 必须要实现

AudioCodec* GetAudioCodec() override {

static GecAudioCodec codec(i2c_bus_, pca9557_); //常见编码器对象

return &codec;

}

Display* GetDisplay() override {

return display_;

}

Backlight* GetBacklight() override {

static PwmBacklight backlight(DISPLAY_BACKLIGHT_PIN,DISPLAY_BACKLIGHT_OUTPUT_INVERT);

return &backlight;

}

private:

i2c_master_bus_handle_t i2c_bus_ = nullptr;     //ic总线的操作句柄

GecPca9557* pca9557_ = nullptr;                 //pca9557的操作句柄

Display* display_ = nullptr;                    //显示屏设备的操作句柄

void InitializeI2c() {

i2c_master_bus_config_t config = {};

config.i2c_port = BOARD_I2C_PORT;

config.sda_io_num = BOARD_I2C_SDA_GPIO;

config.scl_io_num = BOARD_I2C_SCL_GPIO;

config.clk_source = I2C_CLK_SRC_DEFAULT;

config.glitch_ignore_cnt = 7;

config.intr_priority = 0;

config.trans_queue_depth = 0;

config.flags.enable_internal_pullup = true;

ESP_ERROR_CHECK(i2c_new_master_bus(&config, &i2c_bus_));    //创建I2C的句柄i2c_bus_

pca9557_ = new GecPca9557(i2c_bus_,BOARD_PCA9557_ADDR);

ESP_LOGI(TAG,"I2C initialized: port=%d SDA=%d SCL=%d", BOARD_I2C_PORT, BOARD_I2C_SDA_GPIO, BOARD_I2C_SCL_GPIO);

}

void InitializeSpi() {

spi_bus_config_t config = {};

config.mosi_io_num = DISPLAY_SPI_MOSI_PIN;

config.miso_io_num = GPIO_NUM_NC;

config.sclk_io_num = DISPLAY_SPI_SCK_PIN;

config.quadwp_io_num = GPIO_NUM_NC;

config.quadhd_io_num = GPIO_NUM_NC;

/*

* LVGL不会每次传输整屏，但该值需要能够覆盖

* 驱动可能提交的最大像素块。

*/

config.max_transfer_sz =DISPLAY_WIDTH * DISPLAY_HEIGHT *sizeof(uint16_t);

ESP_ERROR_CHECK(spi_bus_initialize(DISPLAY_SPI_HOST,&config,SPI_DMA_CH_AUTO));

ESP_LOGI(TAG, "SPI initialized: host=%d MOSI=%d SCLK=%d", DISPLAY_SPI_HOST,DISPLAY_SPI_MOSI_PIN, DISPLAY_SPI_SCK_PIN);

}

void InitializeDisplay() {

esp_lcd_panel_io_handle_t panel_io = nullptr;

esp_lcd_panel_handle_t panel = nullptr;

/*

* LCD CS由PCA9557控制。

* 先拉低CS，之后LCD作为SPI总线上唯一设备保持选中。

*/

pca9557_->SetOutputState( BOARD_PCA9557_LCD_CS_BIT, false);

esp_lcd_panel_io_spi_config_t io_config = {};

io_config.cs_gpio_num = DISPLAY_SPI_CS_PIN;

io_config.dc_gpio_num = DISPLAY_DC_PIN;

io_config.spi_mode = 0;

io_config.pclk_hz = 20 * 1000 * 1000;

io_config.trans_queue_depth = 10;

io_config.lcd_cmd_bits = 8;

io_config.lcd_param_bits = 8;

ESP_ERROR_CHECK(esp_lcd_new_panel_io_spi( DISPLAY_SPI_HOST, &io_config, &panel_io));

esp_lcd_panel_dev_config_t panel_config = {};

panel_config.reset_gpio_num = DISPLAY_RESET_PIN;

panel_config.rgb_ele_order =LCD_RGB_ELEMENT_ORDER_BGR;

panel_config.bits_per_pixel = 16;

panel_config.data_endian =LCD_RGB_DATA_ENDIAN_BIG;

ESP_ERROR_CHECK(esp_lcd_new_panel_st7789( panel_io,&panel_config, &panel));

ESP_ERROR_CHECK(esp_lcd_panel_reset(panel));

ESP_ERROR_CHECK( esp_lcd_panel_init(panel));

ESP_ERROR_CHECK(esp_lcd_panel_invert_color(panel,DISPLAY_INVERT_COLOR));

ESP_ERROR_CHECK( esp_lcd_panel_swap_xy(panel,DISPLAY_SWAP_XY));

ESP_ERROR_CHECK( esp_lcd_panel_mirror( panel,DISPLAY_MIRROR_X,DISPLAY_MIRROR_Y));

ESP_ERROR_CHECK(esp_lcd_panel_disp_on_off( panel,true));

display_ = new SpiLcdDisplay( panel_io, panel,DISPLAY_WIDTH, DISPLAY_HEIGHT,

DISPLAY_OFFSET_X,

DISPLAY_OFFSET_Y,

DISPLAY_MIRROR_X,

DISPLAY_MIRROR_Y,

DISPLAY_SWAP_XY);

GetBacklight()->RestoreBrightness();

ESP_LOGI(TAG, "LCD initialized: %dx%d",DISPLAY_WIDTH,DISPLAY_HEIGHT);

}

};

//注册板载设备

DECLARE_BOARD(GecEsp32S3Board);

- 修改components/touch_ft6x36/idf_component.yml

- 修改components/lcd_st7789/idf_component.yml

- 从>=5.5.5修改为>=5.5.3

- MCP服务器添加

- main/boards/gec_esp32s3/gec_esp32s3_board.cc

- 添加头文件

#include "mcp_server.h"

#include "stdexcept"

- 在GecEsp32S3Board类中的private添加

- MCP实际调用的配置函数

int SetMicrophoneGain(int gain_db) {

// 检查参数，避免其他调用路径传入无效数值

if (gain_db < 0 || gain_db > 33) {

throw std::invalid_argument(

"gain_db 必须在 0 到 33 之间");

}

if (gain_db % 3 != 0) {

throw std::invalid_argument(

"gain_db 必须是 3 的整数倍");

}

auto codec = GetAudioCodec();

if (codec == nullptr) {

throw std::runtime_error("音频设备尚未就绪");

}

// 调用你的 GecRawCodec，内部会配置 ES7210

codec->SetInputGain(static_cast<float>(gain_db));

// 返回软件记录的已应用增益

return static_cast<int>(codec->input_gain());

}

bool SetLed( bool statu)

{

//通过GPIO设置LED的状态

}

- 添加MCP的功能

//初始化MCP工具

void InitializeTools(){

//获取MCP服务器的访问句柄

auto &server = McpServer::GetInstance();

// server.AddTool(

//     工具名称,

//     工具描述,

//     参数列表,

//     执行回调

// );

server.AddTool(

"self.microphone.set_gain",

"设置本设备麦克风输入增益,单位为dB"

"允许范围为0到33, 必须是3的整数倍."

"用于调整麦克风收音增益, 不用于调整扬声器音量.",

//属性的列表

PropertyList({

//属性

Property(

"gain_db",

kPropertyTypeInteger,

0,

33

)

}),

//参数                             返回值

[this](const PropertyList& properties ) -> ReturnValue {

int gain_db = properties["gain_db"].value<int>();   //MCP服务器返回的增益

int applied_gain = SetMicroPhoneGain( gain_db);     //执行本地功能

ESP_LOGI("GEC_MCP","Microphone gain applied: %d dB",applied_gain);

// 4. 返回结果给调用方

return std::string("麦克风增益已设置为 ") + std::to_string(applied_gain) + " dB";

}

);

server.AddTool(

"self.led.set_statu",

"设置本设备LED灯状态",

//属性的列表

PropertyList({

//属性

Property(

"led_statu",

kPropertyTypeBoolean

)

}),

//参数                             返回值

[this](const PropertyList& properties ) -> ReturnValue {

bool led_statu  = properties["led_statu"].value<bool>();   //MCP服务器返回的增益

bool applied_statu = SetLed( led_statu);     //执行本地功能

ESP_LOGI("GEC_MCP","Led Statu applied: %s ",applied_statu: ON ? OFF);

// 4. 返回结果给调用方

return std::string("LED灯状态一设置为 ") + std::to_string(applied_statu:”打开”?”关闭”) ;

}

);

}

- 类型定义读取

- 布尔值Property("enabled", kPropertyTypeBoolean).value<bool>()

- 整数Property("value", kPropertyTypeInteger).value<int>()

- 字符串Property("mode", kPropertyTypeString).value<std::string>()

- 初始化MCP工具

- main/boards/gec_esp32s3/gec_esp32s3_boards.cc文件中

- GecEsp32Board类中的GecEsp32Board添加

InitializeTools();